# Sample

Snagging Report Saturday, 24 September 2022

Prepared For Anon

95 Defect Defects Identified

Ian Lively MRPSA (Former CEnv FCIOB MCABE MAPM) Lively Professional Services Ltd

Location 1 Sample

Assigned To Identification Summary

Overall the house has been finished to a good standard throughout however with a little more attention to detail the majority of issues highlighted within the report could have been avoided.

Location 2 Front Elevation

Assigned To Groundworker

The manhole needs lowering and the screws fitting in the lid.

Location 3 Roof

Assigned To Roofer

The bottom dry verges need mechanically fixing to comply with BS5534

Location 4 Front Elevation

Assigned To Builder

The turf needs monitoring to see if it comes back or replace.

Location 5 Front Elevation

Assigned To Plastic Surgeon

The front canopy and gallows brackets are marked

Location 6 Front Elevation

Assigned To Groundworker The aco drain needs cleaning.

Location 7 Front Elevatio

Assigned To Roofer

The lead flashing needs patinating.

Location 8 Front Elevation

Assigned To Joiner

The right hand gallows bracket is not plumb.

Location 9 Right Hand Elevation

Assigned To Landscaper

The landscaping needs finishing

Location 10 Right Hand Elevation

Assigned To Roofer

The flashing needs redressing

Location 11 Right Hand Elevation

Assigned To Plastic Surgeon The electric box is damaged

Location 12 Right Hand Elevation

Assigned To Groundworker The air brick is damaged.

Location 13 Right Hand Elevation

Assigned To Groundworker

The electric hockey stick needs a clip

Location 14 Right Hand Elevation

Assigned To Bricklayer

The electric box is no plumb.

Location 15 Rear Elevation

Assigned To Plumber

The low level gutter is holding water suggesting incorrect falls.

Location 16 Rear Elevation

Assigned To Plumber

The gutter is holding water suggesting incorrect falls.

Location 17 Rear Elevation

Assigned To Mastic

Underneath the bifold doors have not been sealed.

Location 18 Hall

Assigned To Plastic Surgeon

The front door glazing bead is chipped.

Location 19 Hall

Assigned To Decorator The wall needs filling.

Location 20 Hall

Assigned To Decorator

The wall needs. Filling to the right of the front door.

Location 21 Hall

Assigned To Decorator

The right hand front door reveal needs filling.

Location 22 Hall

Assigned To Plasterer

The ceiling above the front door is out of tolerance.

NHBC Standards Chapter 9.1.3

Location 23 Hall

Assigned To Joiner

The front door needs tightening on the seals to prevent cold air ingress.

Location 24 Hall

Assigned To Plumber

The radiator needs balancing.

Location 25 Hall

Assigned To Decorator

The garage door needs filling in several places.

Location 26 Hall

Assigned To Joiner

The latch is preventing the fire door to close on the garage door.

Location 27 General

Assigned To Joiner

Insufficient fixings in the door handles.

Location 28 General

Assigned To Decorator

The architrave needs caulking behind the hinges.

Location 29 Study

Assigned To Decorator

The wall needs sanding and filling.

Location 30 Hall

Assigned To Decorator

The wall needs filling between the switches.

Location 31 Downstairs Cloaks

Assigned To Decorator

The door edge needs filling.

Location 32 Downstairs Cloaks

Assigned To Wall Tiller

The sink pipes need grouting.

Location 33 Downstairs Cloaks

Assigned To Wall Tiller

The tiles and architrave need sealing.

Location 34 Downstairs Cloaks

Assigned To Wall Tiller

Underneath the window board needs grouting.

Location 35 Downstairs Cloaks

Assigned To Decorator

The wall needs touching up above the tile trim.

Location 36 Downstairs Cloaks

Assigned To Wall Tiller

The tile needs refitting on the boxing in.

Location 37 Downstairs Cloaks

Assigned To Wall Tiller

The grout needs removing from the tile trim.

Location 38 Downstairs Cloaks

Assigned To Joiner The wall is not plumb.

NHBC Standards Chapter 9.1.3

Location 39 Hall Cupboard

Assigned To Decorator / Electrician

The sockets need levelling and the wall filling.

Location 40 Hall

Assigned To Decorator The ceiling needs filling.

Location 41 Kitchen

Assigned To Joiner

The cut out in the sink base shelf needs sealing to prevent it from blowing if the trap leaks.

Location 42 Kitchen

Assigned To Plastic Surgeon

The infill underneath the microwave is marked.

Location 43 Kitchen

Assigned To Plastic Surgeon

The bottom drawer to the right of the microwave is scratched and chipped.

Location 44 Kitchen

Assigned To Plastic Surgeon

The top of the tall housing door is marked.

Location 45 Kitchen

Assigned To Decorator/ Plasterer

The ceiling needs sanding and filling in several areas.

Location 46 Kitchen

Assigned To Kitchen Fitter

The island unit end panels are not tight against the back board.

Location 47 Kitchen

Assigned To Window Fitter

The strip needs refitting to the bifold door.

Location 48 Utility

Assigned To Plastic Surgeon The tall housing is chipped.

Location 49 Utility

Assigned To Joiner

The cut out in the sink base unit shelf needs sealing to prevent it from blowing if the trap leaks.

Location 50 Utility

Assigned To Decorator The wall needs filling.

Location 51 Utility

Assigned To Cleaner

The door frame needs cleaning behind the hinges.

Location 52 Kitchen

Assigned To Decorator

The bottom of the door is damaged.

Location 53 Lounge

Assigned To Decorator The wall needs painting.

Location 54 Lounge

Assigned To Window Fitter

The patio doors need tightening on the seals to prevent cold air ingress.

Location 55 Lounge

Assigned To Decorator

The skirting board needs caulking behind the radiator.

Location 56 Stairs To First Floor

Assigned To Joiner

The bottom newel is not plumb.

NHBC Standards Chapter 6.6.12

Location 57 Stairs To First Floor

Assigned To Decorator

The bottom of the banister is damaged and the staining needs removing from the newel post.

Location 58 Stairs To First Floor

Assigned To Joiner

The handrail has been poorly cut.

Location 59 Stairs To First Floor

Assigned To Decorator

The infill needs filling and painting.

Location 60 Stairs To First Floor

Assigned To Plastic Surgeon

The top of the handrail is poorly finished.

Location 61 Stairs To First Floor

Assigned To Joiner

The top newel is not plumb.

NHBC Standards Chapter 6.6.12

Location 62 Landing

Assigned To Decorator

The bathroom casing needs filling and the bead glossing.

Location 63 Landing

Assigned To Decorator

The skirting board needs caulking behind the radiator.

Location 64 Bedroom 4

Assigned To Decorator

The trowel marks need sanding.

Location 65 Bedroom 4

Assigned To Mastic

The sealant needs reapplying to the window frame.

Location 66 Bedroom 4

Assigned To Decorator

The wall needs touching up between the windows.

Location 67 Bedroom 4

Assigned To Plasterer

The right hand window head is out of tolerance.

NHBC Standards Chapter 9.1.4

Location 68 Bedroom 4

Assigned To Insulation

The insulation needs adjusting to prevent heat loss.

Location 69 Bathroom

Assigned To Decorator

The architraves needs sanding.

Location 70 Note

Assigned To Plumber

No none return valves fitted to the sink pipes?

Location 71 Bathroom

Assigned To Plastic Surgeon The bath is marked.

Location 72 Bathroom

Assigned To Plasterer

The pipe boxing needs filling out, showing on the mirror.

Location 73 Bathroom

Assigned To Plastic Surgeon The shower tray is scratched.

Location 74 Bathroom

Assigned To Electrician

The extractor cover needs refitting.

Location 75 Bedroom 3

Assigned To Decorator

The casing needs caulking and filling.

Location 76 Bedroom 3

Assigned To Decorator

The window head needs sanding.

Location 77 General

Assigned To Electrician

The light pendant covers need fitting.

Location 78 Bedroom 5

Assigned To Decorator The wall needs filling.

Location 79 Bedroom 5

Assigned To Decorator

The paint run needs sanding on the skirting board.

Location 80 Bedroom 5

Assigned To Decorator

The skirting board needs caulking behind the radiator.

Location 81 Bedroom 5

Assigned To Decorator

The door stop bead needs filling.

Location 82 Landing

Assigned To Decorator

Bedroom 3 & 5 architrave edges need painting.

Location 83 Cylinder Cupboard

Assigned To Joiner

The pin needs punching in the architrave.

Location 84 En Suite

Assigned To Plasterer The ceiling is running out

Location 85 En Suite

Assigned To Decorator

The top of the skirting board needs caulking.

Location 86 En Suite

Assigned To Wall Tiller

The pin holes in the shower need grouting.

Location 87 Bedroom 1

Assigned To Decorator

The door stop bead needs sanding.

Location 88 En Suite

Assigned To Decorator The door needs sanding.

Location 89 En Suite

Assigned To Decorator

The wall needs sanding around the radiator pipes and the skirting board needs caulking.

Location 90 En Suite

Assigned To Decorator

The ceiling needs sanding and touching up.

Location 91 Landing

Assigned To Joiner The floor is squeaking.

Location 92 Roof

Assigned To Joiner

No walkway to maintain the extractor.

NHBC Standards Chapter 8.3.6

Location 93 Roof

Assigned To Labourer

The extractor duct needs insulating.

Location 94 Rear Elevation

Assigned To Window Fitter

The bifold door handle is chipped.

Location 95 Landing

Assigned To Decorator

The cylinder cupboard door needs touching up